Kawneer Thermal Entrance Door

AA®250 & AA®425

Door Instructions

Placing Door

- Copy the door group and paste it into your project then ungroup the group.

  - When pasting the group, a warning window describing duplicate types may pop up.  Click “Ok” for the group to be pasted in.

- Check on the parameter “Automatically Embed” in the curtain wall type properties.

- Place the door into its position.

Changing Door Width

- Drag the curtain wall to the desired width.

  - Dragging the width of the door pass the maximum or below the minimum will display a constraint error window.  Please see the max/min door width in the door instance properties under the “Constraints” category.

Changing Double Door Height

- Select curtain wall type containing the door you want.  In its type properties, create a duplicate type and rename the height stated in the name to the height you want.

  - Ensure the new height is within the max/min door height in the door instance properties.

- Go into the type properties of the new curtain wall type.  Change the value in the “Spacing” parameter to the desired height; “Spacing” is located below the “Horizontal Grid Pattern” category.

  - Add an additional 1” to desired height to account for the mullion thickness.

    - If desired height is 90”, type 91”.

Changing Single Door Height

- Select curtain wall type containing the door you want.  In its type properties, create a duplicate type and rename the height stated in the name to the height you want.

  - Ensure the new height is within the max/min door height in the door instance properties.

- Go into the type properties of the new curtain wall type.  Change the value in the “Spacing” parameter to the desired height; “Spacing” is located below the “Horizontal Grid Pattern” category.

- Go into the type properties of the new curtain wall type.  Change the value in the “Spacing” parameter to the desired height; “Spacing” is located below the “Horizontal Grid Pattern” category.

  - Add an additional 2 1/4” to desired height to account for the mullion thickness.

    - If desired height is 90”, type 92 1/4”.

- Select the curtain wall.  Then click on the height dimension and type in the height in step 3.

Changing Transom Height

- Drag the top of the curtain wall to desired height.

Changing Door Swing Angle

- If the door is “Existing”, the door swing will stay locked at 45 degrees.

  - The parameter “Existing” is located in the instance property of the door under the “Phasing” category.

- If the door is not “Existing” then the swing angle can be changed.

Creating Detail Views

- Copy and paste the drafting views from the given project and paste them into your project.

- Create a callout and check the “Reference other view” box.  Then select the correct drafting view.

  - Refer to callouts from the given project to reference the correct views.